Collector Ring Signal

Regular Signal

| Plot | Collector | Standard |
| --- | --- | --- |
| Time | - obvious peak at cut - obvious peaks at edges of corrosion - perhaps an above-baseline signal between corrosion peaks | - obvious peak at cut - less cyclic over region of corrosion (ie/ “messier”) |
| Frequency | - most power in 0-3 Hz region | - most power in 0-10 Hz |
| Spectrogram | - bright patches over cut and edges of corrosion | - larger peak at cut - shift towards higher frequency over corrosion |

To do (26/10/22)

- Insert new data into power point

- Remove 3 Hz noise from system

  - Adjust RPM of motor (reduce to 50% of current)

  - Add in extra wheels or change length to reduce “hill” effect

MFL v2 next steps

- When moving MFL v2, it slides over “corroded” bit

  - Roughen up surface

  - Spray with water to help corrosion

- Signal doesn’t look very smooth

  - Increase sample rate of hall effect sensors

  - Get a better ADC (one from Raspberry pi?)

- Collector ring near saturation at all times (sits around 12,000 / 65,000)

  - Switch magnets around to reverse field

    - Just saturates the other way

  - Weaker magnets

  - Greater separation between conductor and magnets

- Improve SNR

  - Move sensor closer to magnets